FrED MOBO — Power-log CSV requirements

What the power-measurement PCB must write so the optimisation software can read it

Rev. 1 — 21 September 2026  ·  For the PCB team  ·  Source: FrED MOBO architecture record v1.4, §2.4 and §9.5  ·  Companion file: power_log_example.csv

# 1.  What the file is for

The optimisation software runs a campaign of short extrusion runs (about three minutes each) and, for every run, needs one number from your board: the mean total electrical power drawn by the three actuators — heater, spooler motor, extruder stepper — over a steady-state window of tens of seconds. That number is the second objective of the optimisation, so any bias in it goes straight into the result.

Your logger is a separate microcontroller with its own clock and its own file. It does not need to talk to the control computer and it does not need to be synchronised with the device log. Alignment to about one second is enough, and the software gets it from a marker you write when the operator presses a button (F6). The operator starts and stops both loggers by hand.

The software computes P(t) = Σ over channels of V(t)·I(t) from your columns, averages it over the window, and stores the result with the run. It never assumes a voltage, never interpolates, and never fills a gap — which is why the conventions below matter.

# 2.  Channels and sensing points

Three channels, each with voltage and current, each sensed on the supply side of its driver — the wires feeding the driver, not the motor coil or the heater element. Supply-side sensing is intended: the objective is the power drawn from the supply, driver losses included, and it is what makes the stepper reading meaningful (a coil-side reading of a chopper-driven stepper would be wrong in a way nothing downstream could detect). The stepper channel has already been confirmed supply-side by the project lead (21 Sep 2026).

| Channel | What it drives | Sensing point | Note |
| --- | --- | --- | --- |
| Heater | Cartridge/band heater on the nozzle block | Supply side of the heater switch (MOSFET/SSR) | PWM-switched. See F4 — this channel is why interval averaging is mandatory. |
| Spooler | DC motor of the take-up spool | Supply side of the motor driver | Load varies with fibre tension; current is the quantity of interest. |
| Extruder | Stepper pushing the preform | Supply side of the stepper driver (confirmed) | Chopper driver: supply-side current is smooth, coil-side is not. |

# 3.  File requirements

MUST = the software cannot read the file, or the number is wrong, without it. SHOULD = strongly preferred; a deviation is workable but costs accuracy or operator effort. MAY = useful if cheap. Where a MUST is impossible, say so in section 5 rather than approximating it — the software has fallbacks for some of them, and needs to know which to switch on.

| ID | Level | Requirement | Why |
| --- | --- | --- | --- |
| F1 | MUST | Plain CSV, one header row, one dialect Plain text (UTF-8 or ASCII, no byte-order mark), the header on line 1, then one row per logging interval. No comment lines, no blank lines, no units row. Use `;` as the separator and a decimal comma (12,006) — the same dialect as the device log from the control computer, so both files open identically in Excel on the same machine. If that is impossible, , separator with . decimal is accepted; whichever it is, use it consistently and say which in section 5. | The parser reads columns by name and takes the dialect from the config; a mixed or undeclared dialect is the one thing it cannot recover from. |
| F2 | MUST | Columns, by exact header name Time (s); Heater voltage (V); Heater current (A); Spooler voltage (V); Spooler current (A); Extruder voltage (V); Extruder current (A); Event. Order is free; names are matched exactly (case and spaces included). Extra columns are ignored. | Voltage next to current per channel, because the supply voltage moves with load and the software must not assume one (record §2.4, request (b)). |
| F3 | MUST | Time base: elapsed seconds since the logger started A decimal number of seconds, at least 3 decimals, starting at or near 0, strictly increasing, never resetting inside a file. A fixed logging interval (F5). | The software aligns your file to the device log by an offset read from the markers (F6); any monotonic elapsed-time base works, a wall-clock base is not required. |
| F4 | MUST | Each row is the mean over its logging interval, not one instantaneous sample Sample the ADC fast — at least 1 kHz per channel — and write, for each row, the average of the samples taken during that interval. Do not write the last sample, the peak, or a single reading taken at the start of the interval. If the board can compute it, the average of the instantaneous product V·I over the interval is even better than the averages of V and I separately (MAY, F10). | The heater is PWM-switched — the control firmware runs it at about 1 Hz with duties of a few percent — so a logger taking one reading every 100 ms would alias against the PWM and read the heater current as anything between 0 and the full ON current. Averaging over the interval makes the row equal to duty × I_on, which is the physical mean. This is the single requirement most likely to be missed, and the error it causes is invisible in the file. |
| F5 | SHOULD | Logging interval 100 ms (10 Hz); MUST be at most 1 s 10 Hz gives 300+ rows over a 30 s window and a trace the operator can overlay on the device log at intake. Slower is acceptable down to 1 Hz; faster is unnecessary. | Both objectives are window means over tens of seconds (record §2.4); nothing needs per-sample synchronisation. |
| F6 | MUST | Run markers in the Event column Event is empty on every row except the row during which the operator pressed the marker button: write RUN_START on the first press and RUN_STOP on the second. A second RUN_START before a RUN_STOP restarts the run. Nothing else goes in the column. (MAY: append a counter, RUN_START 7, if the board keeps one.) | The marker is what turns the alignment offset from an estimate into a reading. If a button cannot be added, say so in Q7 — the software then falls back to file timestamps and a manual slide on the intake plot, both of which must be working before the campaign. |
| F7 | MUST | A missing or invalid sample is an empty cell If a channel has no valid reading for an interval, leave that cell empty (12,004;;11,992). Never write 0, -1, 9999, NaN, nan or any other sentinel. Never drop the row: the other channels and the time base stay. | A 0 in the average is a bias; an empty cell is counted and reported at intake, exactly as the device log's temperature drop-outs are. |
| F8 | MUST | Units and resolution Volts and amperes as plain decimals — no unit suffixes, no thousands separators, no scientific notation. At least 3 decimals (1 mV, 1 mA in the file). Full scale must cover each channel's maximum (Q2) with the sensor's actual resolution stated in Q9; the file's 3 decimals are a formatting floor, not a claim of accuracy. | The heater's mean current is a few percent of its ON current, so a 10 mA quantisation on a 250 mA mean is a 4 % error on the heater power. |
| F9 | MUST | One file per logger session; a name that cannot collide The operator starts the logger before the device log and stops it after; the markers delimit the run inside it. One file per session, never appended to a previous one. Name power_<boardid>_<NNNN>.csv with a counter that survives power cycles, or power_<boardid>_<YYYYMMDD_HHMMSS>.csv if the board has a clock (Q5). | The software copies the file into the campaign folder under its own name and keeps the original name in the record; two files with the same name would be indistinguishable. |
| F10 | MAY | Extras that help Heater power (W), Spooler power (W), Extruder power (W): the interval mean of the instantaneous product V·I, if the board can compute it (used in preference to V̄·Ī when present). Wall clock in ISO 8601 if there is a real-time clock. A separate small text file per board stating firmware version, shunt values, gains and calibration date. | Product-mean vs mean-of-products only differs when V and I vary together inside an interval, which on a PWM channel they can; the rest is provenance for the paper. |

# 4.  Example — power_log_example.csv

The companion file is 15 s of log at 10 Hz in exactly the required shape: markers at 2.0 s and 12.0 s, one deliberately empty cell at 5.7 s, ; separator, decimal comma. The numbers are illustrative — a 12 V rail is assumed on all three channels and the currents are invented; only the shape is a requirement. The rows below are copied from it.

| Time (s) | Heater voltage (V) | Heater current (A) | Spooler voltage (V) | Spooler current (A) | Extruder voltage (V) | Extruder current (A) | Event |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0,0000 | 12,006 | 0,246 | 11,982 | 0,299 | 11,980 | 0,600 |  |
| 0,1000 | 11,990 | 0,269 | 12,010 | 0,304 | 12,002 | 0,596 |  |
| 0,2000 | 12,014 | 0,254 | 11,991 | 0,300 | 11,974 | 0,593 |  |
| … | … | … | … | … | … | … | … |
| 1,9000 | 12,025 | 0,295 | 11,999 | 0,296 | 11,997 | 0,596 |  |
| 2,0000 | 11,989 | 0,319 | 11,984 | 0,298 | 11,974 | 0,605 | RUN_START |
| 2,1000 | 12,019 | 0,299 | 11,986 | 0,302 | 11,989 | 0,600 |  |
| … | … | … | … | … | … | … | … |
| 5,7000 | 12,004 |  | 11,992 | 0,295 | 12,004 | 0,595 |  |
| 5,8000 | 12,016 | 0,224 | 11,962 | 0,298 | 12,012 | 0,605 |  |
| … | … | … | … | … | … | … | … |
| 12,0000 | 11,992 | 0,229 | 11,996 | 0,295 | 11,991 | 0,603 | RUN_STOP |
| 12,1000 | 11,998 | 0,257 | 12,013 | 0,301 | 11,977 | 0,597 |  |
| … | … | … | … | … | … | … | … |
| 14,9000 | 11,994 | 0,233 | 11,993 | 0,301 | 11,995 | 0,594 |  |

Read with pandas as pd.read_csv(path, sep=';', decimal=','): every numeric column parses as float64, Event as text with two non-empty rows, and the one empty cell becomes a missing value that is counted, not zero-filled. Mean total power over the marked run in this example: 13.64 W over 101 rows.

# 5.  Questions for the PCB team

Type into the yellow cells. Partial answers and 'unknown yet' are useful; blanks are not. These go into the software's configuration and into the paper's description of the measurement.

| Question | Answer |
| --- | --- |
| Q1 Supply voltage of each channel (heater, spooler, extruder), nominal and range under load. | Q1: [ PCB team: type here ] |
| Q2 Expected maximum current per channel, and the full-scale range of each current sensor. | Q2: [ PCB team: type here ] |
| Q3 Heater PWM frequency and where it is generated (control firmware? your board?). The architecture record assumes about 1 Hz — please confirm or correct. | Q3: [ PCB team: type here ] |
| Q4 ADC sample rate per channel and how the per-row value is formed (mean over the interval? see F4). If the board cannot average, say so. | Q4: [ PCB team: type here ] |
| Q5 Does the board have a real-time clock? If yes, is it set, and how? | Q5: [ PCB team: type here ] |
| Q6 Where does the file go — SD card, USB mass storage, serial to a PC — and how does the operator get it after a run? | Q6: [ PCB team: type here ] |
| Q7 Can a marker button (F6) be wired? If not, what alternative marker is possible? | Q7: [ PCB team: type here ] |
| Q8 Firmware version string, and where it is recorded (file name? separate file?). | Q8: [ PCB team: type here ] |
| Q9 Current-sense method per channel (shunt value and gain, or Hall sensor and part number), resolution in mA, and the calibration procedure and date. | Q9: [ PCB team: type here ] |
| Q10 Anything the board cannot do from section 3, and any known limitation (bandwidth, self-heating, common ground issues). | Q10: [ PCB team: type here ] |

# 6.  What the software does with the file

Reads columns by name through a configuration mapping, so a renamed header is a one-line config change — but only if the file is otherwise as specified (F1, F3, F7).

Aligns your elapsed time to the device log's elapsed time using the RUN_START marker (F6). Fallbacks, in order: file creation timestamps; the operator sliding the two traces on the intake plot. The offset is stored with the run.

Computes P(t) = V_h·I_h + V_s·I_s + V_f·I_f per row (or uses your power columns if F10 is provided), averages over the steady-state window chosen from the diameter trace, and stores the mean, the number of rows used, and the number of empty cells.

Records the power source as PM (measured) with the file's name and your firmware version, so the paper can state exactly how f₂ was measured.

Never interpolates across empty cells, never assumes a voltage, never rescales a channel. Anything that looks like a sensor fault is reported at intake, not corrected.